# RetroLogic Project: Initial Progress Report

This report summarizes the initial implementation progress for the RetroLogic project, focusing on the core VGA controller development and the establishment of the project's foundational structure.

## 1. Hardware Architecture Definition

Detailed hardware architecture and component selection for the RetroLogic board have been defined. The design centers around the Lattice iCE40UP5K FPGA due to its affordability, open-source toolchain compatibility, and sufficient logic capacity for retro computing applications. Key peripherals include:

- VGA Connector: For video output, chosen for its simplicity of implementation and retro compatibility.

- 3.5mm Audio Jack: For basic audio output using a simple DAC.

- PS/2 Connectors: For keyboard and mouse input, offering direct FPGA interfacing.

- MicroSD Card Slot: For program and data storage.

- Pmod Compatible Connectors: For modular expansion and easy integration of additional peripherals.

- USB-C: For power and programming, utilizing an FTDI chip (e.g., FT2232H) for a single-cable solution.

- LEDs, Push Buttons, DIP Switches: For basic user feedback and debugging.

A conceptual block diagram illustrating these components and their interconnections has been created.

## 2. VGA Controller Verilog IP Development

The core VGA controller Verilog module (vga_controller.v) has been developed to generate a 640x480 resolution @ 60Hz VGA signal. The module includes:

- Timing Parameters: Configured for standard 640x480 @ 60Hz VGA timing, including horizontal and vertical display, front porch, sync pulse, and back porch values.

- Counters: Horizontal and vertical counters to track pixel and line positions.

- Sync Signals: Generation of hsync (horizontal sync) and vsync (vertical sync) signals.

- Video Enable: A video_on signal to indicate the active display area.

- Pixel Coordinates: Outputs pixel_x and pixel_y to provide current pixel coordinates for drawing logic.

A corresponding Verilog testbench (vga_controller_tb.v) was also created to simulate the VGA controller. The simulation was run using Icarus Verilog, and a VCD (Value Change Dump) waveform file (vga_controller.vcd) was generated, allowing for visual verification of the timing and signal integrity.

## 3. Initial Project Structure

A basic project directory structure for RetroLogic has been established:

RetroLogic/

├── hardware/             # For hardware design files (schematics, PCB layouts)

├── verilog/              # For Verilog IP cores and modules

│   └── vga_controller/   # Contains vga_controller.v, vga_controller_tb.v, vga_controller.vcd

├── docs/                 # For project documentation, tutorials, and learning resources

└── tools/                # For scripts and configurations for open-source toolchains

The developed VGA controller files and the simulation waveform have been placed in the verilog/vga_controller/ directory. A preliminary README.md file has also been created at the root of the project to introduce the RetroLogic platform and its structure.

## Next Steps

The next phase will involve further development of core IP, starting with an audio output module, and continuing to build out the educational content and documentation.